DFT Lab : Clock Controllability and Scan Chain Design

Sequential Shift-and-Add Multiplier

Background : Your team's 8-bit sequential multiplier works perfectly in functional simulation. However, the power team has requested clock gating optimization, and the DFT team needs full scan insertion for manufacturing test. Your multiplier passes all functional tests, but the POWER TEAM complains:

"The bit_counter burns power even when the multiplier is IDLE. You're clocking a counter 24/7 that only needs to run during the 8 SHIFT cycles. Add clock gating!"

You add this power optimization to bit_counter:

// Power-optimized version (BROKEN for DFT!)

wire gated_clk = clk & count_en;  // Only clock when counting

always @(posedge gated_clk) begin

if (rst || clear) count <= 0;

else count <= count + 1;  // removed count_en check

end

Design a DFT Compliant Solution

Objective : Add 2 independent DFT control signals:

Signal 1: test_mode

Purpose: Bypass clock gates (keep clocks free-running during test. When active: Entire test session)

Signal 2: scan_enable

Purpose: Switch data path between functional and scan inputs. When active: alternates between Shift and Capture modes

Requirements:

1. Modify bit_counter to accept both test_mode and scan_enable:

module bit_counter (

input clk, rst, clear, count_en,

input test_mode,     // New: Bypass clock gating

input scan_enable,   // New: Shift vs. capture mode

input [3:0] scan_in, // New: Scan chain input

output reg [3:0] count,

output last

);

2. In Functional mode (test_mode=0, scan_enable=0):

- Clock gating active (power savings)

- Normal counting behavior

3. In DFT mode with SHIFT (test_mode=1, scan_enable=1):

- Clock gating BYPASSED (full controllability)

- Flip-flops load from scan_in (ignore count_en/clear)

4. In DFT mode with CAPTURE (test_mode=1, scan_enable=0):

- Clock gating BYPASSED

- One functional clock cycle applied

- Flip-flops capture functional next-state

5. Use the provided ICG (Integrated Clock Gate) cell:

module ICG (

input clk,

input enable,       // Functional enable (count_en)

input test_enable,  // Test mode (bypass)

output gated_clk

);

// Latch-based clock gate

reg en_latch;

always @(clk or enable or test_enable)

if (!clk) en_latch <= enable | test_enable;

assign gated_clk = clk & en_latch;

endmodule

YOUR TASKS:

Task B1: Modify bit_counter.v

- Instantiate ICG with proper connections

- Add scan mux: count <= scan_enable ? scan_in : count_next_func

- Ensure test_mode bypasses clock gating

Task B2: Modify mult_regs.v

- Add scan_enable and scan_in ports for A[8:0], M[7:0], Q[7:0]

- Add scan mux for each register

- Maintain functional load/add/shift operations when scan_enable=0

Task B3: Modify controller.v

- Add scan_enable and scan_in[2:0] for state register

- Expose state[2:0] as output for scan chain

- Add scan mux for state register

Task B4: Modify seq_multiplier.v (top-level)

- Wire the scan chain:   scan_in  controller.state[2:0] mult_regs.A[8:0] mult_regs.M[7:0] mult_regs.Q[7:0] bit_counter.count[3:0] scan_out

- Total chain length: 3 + 9 + 8 + 8 + 4 = 32 bits

- Add test_mode, scan_enable, scan_in, scan_out ports

Scan Chain Order:

scan_in  controller.state[2:0] mult_regs.A[8:0] mult_regs.M[7:0] mult_regs.Q[7:0] bit_counter.count[3:0] scan_out

Part C: Create DFT Aware Testbench

Write a comprehensive testbench that proves your solution works:

Test C1: Functional Test (test_mode=0, scan_enable=0)

- Run the original 13x11 multiplication

- Monitor clock toggles on gated_clk vs. system clk

- Verify: gated_clk stops toggling when FSM is in IDLE

- Verify: product = 143 (functional correctness maintained)

- Measure: Power savings (count clock toggles)

Test C2: Scan Shift Test (test_mode=1, scan_enable=1)

- Force test_mode=1 (bypass clock gating)

- Set scan_enable=1 (enter shift mode)

- Shift in a 32-bit test pattern over 32 clock cycles

- Test pattern example:

state = 3'b100 (SHIFT state)

A = 9'h055

M = 8'hAA

Q = 8'h33

count = 4'h5

- After 32 cycles, set scan_enable=0

- Verify all registers contain the shifted pattern

- Key assertion: Shift succeeded even though count_en=0 (proves test_mode works)

Test C3: Shift-Capture-Shift Test (full ATPG pattern)

Step 1: SHIFT IN (scan_enable=1, test_mode=1)

- Shift in pattern: state=LOAD, A=0, M=13, Q=11, count=0

- Takes 32 clock cycles

Step 2: CAPTURE (scan_enable=0, test_mode=1)

- Apply ONE functional clock cycle

- FSM should transition: LOAD â CHECK

- Registers update per functional logic

Step 3: SHIFT OUT (scan_enable=1, test_mode=1)

- Shift out the captured response over 32 cycles

- Compare against expected "golden" response

- Verify: state=CHECK, A=0, M=13, Q=11, count=0

Test C4: At-Speed Path Test (test_mode=1)

- Use scan to load: count = 4'b0110 (6)

- Set scan_enable=0 (capture mode)

- Apply one clock with count_en=1

- Measure propagation delay: count â last â FSM

- Verify: count increments to 7, last asserts to 1

- Compare timing: test_mode=0 vs test_mode=1 (should be identical)

Required Assertions:

1. assert(gated_clk toggles when test_mode=1)

else $error("Clock gating not bypassed");

2. assert(count shifts when scan_enable=1 && count_en=0)

else $error("Scan shift failed");

3. assert(FSM transitions correctly in capture cycle)

else $error("Functional capture failed");